Permeability Dimensions & Units

Permeability is a measure of the fluid conductivity of a rock.

From Darcy’s Equation, the dimension of permeability is

k=qmLADp=L3T⋅P⋅T1⋅L1⋅1L2⋅1P=L2

The permeability unit is defined in an older system now called Darcy Units.  Other unit systems are shown in braces and brackets:  {SI Units} and [Oilfield Units]

qsA −kμdpds+ρgcdZds −kμdΦds ;where Φ=p−ρgcZ; Z+↓

k = permeability, d {m2} [md]

qs = volumetric flow rate along flow path, cm3/s {m3/s} [barrel/d]

(42 US gallons/barrel) (5.6145833 ft3/barrel)

A = cross-sectional area of flow path, cm2 {m2} [ft2]

 = fluid viscosity, cp {Pas} [cp]

p = pressure, atm {Pa} [psi]

s = distance along flow path, cm {m} [ft]

 = density, g/cm3 {kg/m3} [lbm/ft3]

g = acceleration due to gravity, 981 cm/s2 {9.81 m/s2} [32.17 ft/s2]

c = gravity term conversion constant, 1.01325 x 106 dyne/cm2atm

{1 Pa/(N/(m2)} [(32.174 lbmft/ lbf s2)(144 in2/ft2)]

Z = vertical elevation measured from datum, cm {m} [ft]

Note:  Darcy’s Equation in oilfield units is non-coherent, and requires an additional unit conversion factor (not shown)