Petroleum Engineering 311, Spring 2003

Homework #6

Wednesday, 2/26/03

Due: 		Monday, 3/3/03

Format:  	Work out on Engineering Analysis Pad per instructions in the syllabus.  State any necessary assumptions

Problem:

1.  Use Excel to calculate and graph the Leverett J-Function, J(Sw) for the following laboratory  data.  Remember that J(Sw) is dimensionless and to work in consistent units.

| ### Sample | 10A | 48D | 77C | Other Lab Data |
| --- | --- | --- | --- | --- |
| Depth | ### 9304.1 ft | 9342.6 ft | 9374.8 ft | ##### Fluid System: Air-Water |
| k(md) | 45 | 6.3 | 11 |  = 0 degrees |
|  | 0.197 | 0.129 | 0.133 |  = 72 dynes/cm |

| Lab Pc (psia) | Sample 10A (Sw) | Sample 48D (Sw) | Sample 77C (Sw) |
| --- | --- | --- | --- |
| 0.0 | 1.0 | 1.0 | 1.0 |
| 1.0 | 1.0 | 1.0 | 1.0 |
| 2.0 | 0.820 | 0.941 | 1.0 |
| 5.0 | 0.541 | 0.581 | 0.726 |
| 10.0 | 0.381 | 0.443 | 0.527 |
| 25.0 | 0.285 | 0.279 | 0.410 |
| 50.0 | 0.228 | 0.221 | 0.314 |
| 100.0 | 0.179 | 0.181 | 0.238 |
| 200.0 | 0.152 | 0.144 | 0.191 |
| 500.0 | 0.125 | 0.094 | 0.138 |
| 1000.0 | 0.115 | 0.057 | 0.109 |

HINT:  You may copy and paste these data from the website.

2.  Use Excel to calculate and graph Log pc vs Log Sw* for Samples 10A & 48D above.  Determine Swi and  and the power equation best fitting the data.